# METHOD OF STATEMENT

Reference No: EW/Q/0946/23

Date: 17th July 2023

Description of work

To unload 1 unit machine from 1 x 40ft container, shift and position into production area at ground floor

Machine Spec

1 unit Samson SMV-1060L CNC

Dim: 330 x 228 x 255 cm

Wt: 6, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

20/07/23

Time of Operation

09:45 am

Location of Operation

#### 2 Gul St 2

Supermec Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Hand jack

Woodblock

Wire/webbing sling

1 x platform (to lay out for machine pathway)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry back to back with container

- Crane to pull out machine pallet from container onto lorry chassis

- Unscrew all nuts from

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175