# METHOD OF STATEMENT

Date: 14th June 2023

Description of work

To unload 1 unit machine from 1 x 40ft container, shift and position into production area at ground floor

Machine Spec

1 unit L & L LG63X1000 CNC Lathe

Dim: 4851 x 2810 x 2950

Wt: 16, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

Est: 21/06/23

Time of Operation

09:30 am

Location of Operation

#### 21 Loyang Lane

Windsor Airmotive Asia Pte Ltd

Equipment Use

1 x 50ton mobile crane

1 x 633 lorry loader

Rollers/pallet truck

Hydraulic/hand jack/woodblock

Wire/webbing sling

Shackle

1 x forklift

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy mobile/lorry crane at designated unloading area

- Reverse lorry back to back with container

- Crane to pull out machine base from container onto lorry chassis

- Attached sling to mobile crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Use of forklift to assist in pulling/pushing machine

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175