# METHOD OF STATEMENT

Date: 19th Apr 2024

Description of work

To unload  cases of 5 unit machine & accessories from 5 x 40ft HQ container, uncrate, shift and position into production area at ground floor

Machine Spec

Mazak VCN-430AL

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

04/24

Time of Operation

09:45 am

Location of Operation

#### 40 Lok Yang Way

Shine Precision Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Hand jack

Woodblock

Wire/webbing sling

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane to near container

- Attached sling to crane hook

- Secured to rigging point of case

- Hoist up case from container and lower down to the ground

- Uncrate case and put aside

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Using of forklift to fork down smaller item

- Uncrate case

- Shift and position into factory premise

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175