# METHOD OF STATEMENT

Date: 15th May 2023

Description of work

1) To unload 1 unit machine from 1 x 40ft container, uncrate, shift and position into production area at ground floor

2) To shift out 1 unit used 2.5ton Valve Tester machine, load up onto lorry chassis and deliver to scrapyard for disposed

Machine Spec

1 unit Valve Test machine

Dim: 475 x 213 x 201 cm

Wt: 5, 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 9 Gul Rd #01-01

Emerson Asia Pacific Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Hand jack/crowbar/woodblock

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry back to back with container

- Crane to pull out machine from container onto lorry chassis

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175