# METHOD OF STATEMENT

Date: 17th Oct 2025

Description of work

To unload 13 packages of 1 unit machine with accessories from 1 x 40ft HQ & 1 x 20ft GP container, uncrate, shift and position into production area at ground floor

Machine Spec

1 unit G-V1540B Double Column Machining Center

Refer to attached packing list

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

18/10/25

Time of Operation

09:00 am

Location of Operation

#### 1 Tuas Ave 12

Completion Products Pte Ltd

Equipment Use

1 x 50ton mobile crane

1-2 x 633 lorry loader

Rollers/pallet truck

Hydraulic jack

Woodblock

Wire/webbing sling

Shackle

Canvas/metal plate (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deployment of mobile/lorry crane into site

- Reverse lorry crane back to back with container

- Set up crane with outriggers fully extended and resting on steel plate/timber mats as base plate

- Lorry crane to pull out packages from container onto lorry chassis

- Mobile crane to hoist down packages from lorry chassis to the ground

- Uncrate case and put aside

- Riggers to secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing crane and lower down machine to the ground

- Engineer to install machine outside factory

- Insert rollers under machine bed

- Shift and transport machine into factory premise

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175