# METHOD OF STATEMENT

Date: 19th Apr 2024

Description of work

To unload 3 packages of 1 unit machine from 1 x 20ft GP container, uncrate, shift and position into production area at ground floor

Machine Spec

3 Packages of 1 unit D-28 CMM machine

Main machine Dim: 88” x 60” x 90” / Wt: 1, 215 kg

Accessories Dim: 37” x 20” x 28”

Steel stand Dim: 28” x 16” x 20”

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

Est: 14-15/05/24

Time of Operation

09:45 am

Location of Operation

#### 31 Tuas South Ave 10

Energy Alloys Pte Ltd

Equipment Use

1 x 633 lorry loader

1 x 3ton forklift

Pallet truck

Crowbar/woodblock

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry back to back with container

- Crane to pull out machine skid from container onto lorry chassis

- Using forklift to fork down skid to the ground

- Unscrew all bolt and nut

- Using forklift to detach machine from base

- Load machine onto steel stand

- Insert pallet truck under machine stand

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175