# METHOD OF STATEMENT

Date: 30th Dec 2024

Description of work

To unload machine and accessories from lorry chassis, uncrate, shift and position into production area at ground floor

Machine Spec

Refer to attached

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

30/12/24

Time of Operation

10:00 am

Location of Operation

#### 37 Joo Koon Rd

Energy Pipeline Services Pte Ltd

Equipment Use

2-3 x 633 lorry loader

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry crane into inside factory premise

- Using crane to pull out machine from container onto l

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine to location

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175