# METHOD OF STATEMENT

Date: 16th Jan 2025

Description of work

To unload 2 skid of 1 unit machine from 1 x 40ft HQ container, shift and position into production area at ground floor

Machine Spec

RXP 500 machine

Skid 1: 265 x 230 x 255 cm / Wt: 6, 000 kg

Skid 2: 265 x 230 x 165 cm / Wt: 571 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

05/02/25

Time of Operation

09:45 am

Location of Operation

#### 51 Joo Koon Circle

Omni Mold Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Hand jack/crowbar

Woodblock

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry crane into site premise

- Reverse lorry crane back to back with container

- Crane to pull out machine skid from container onto lorry chassis

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from skid

- Swing crane and lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175