# METHOD OF STATEMENT

Date: 04th Mar 2024

Description of work

To unload 4 cases of 2 machine & accessories from 2 x 40ft HQ container, uncrate, shift and position into production area at ground floor

Machine Spec

2 unit Smart CNC Turning Center / NL6000S

Box 1: 4800 x 2300 x 2550 mm / Wt: 12, 150 kg

Box 2: 3950 x 2300 x 1750 mm / Wt: 1, 930 kg

Box 1: 4800 x 2300 x 2550 mm / Wt: 12, 150 kg

Box 2: 3950 x 2300 x 1750 mm / Wt: 1, 930 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

12/03/24

Time of Operation

10:00 am

Location of Operation

#### 3 Joo Koon Circle

AME International Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x 16ton forklift

Rollers/pallet truck

Hydraulic/hand jack

Woodblock

Wire/webbing sling

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

# JOB SCOPE

- Reverse lorry crane back to back with container

- Crane to pull out machine case from container onto lorry chassis

- Using 16ton forklift to fork down case to the ground

- Uncrate case and put aside

- Using forklift to fork and detach machine from base

- Transport machine to door entrance

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175