# METHOD OF STATEMENT

Date: 18th Apr 2024

Description of work

To unload 3 packages of 1 unit of machine from lorry chassis, uncrate, shift and position into production area at ground floor

Machine Spec

Sheet Metal Sander machine

Dim 1: 370 x 170 x 230 cm / Wt: 3, 800 kg

Dim 2: 111 x 115 x 235 cm / Wt: 450 kg

Dim 3: 105 x 93 x 120 cm / Wt: 280 kg

Risk Assessment

A copy of Risk Assessment will be attached

Schedule of Operation

30/04/24

Time of Operation

10:00 am

Location of Operation

#### 1 Changi North St 2

ETLA Limited

Equipment Use

1-2 x 633 lorry loader

Rollers / pallet truck

Hand jack / crowbar

Wire sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Hoist down case to the ground

- Uncrate case and put aside

- Rigger will attached required sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175