# METHOD OF STATEMENT

Date: 5th September 2024

Description of work

To unload 1 unit machine from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 case of 1 unit Glory Scan 8.10.6 CMM

Case Dim: 2938 x 1562 x 2502mm/ Gross Weight: 1,804kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

06/09/2024

Time of Operation

09.45am

Location of Operation

#### 1 Pandan Crescent

Emerson Asia Pacific Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire sling/shackle

Crowbar/woodblock

Canvas/metal sheet (to lay out for rubber tiles protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case and put aside

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and upright machine

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175