# METHOD OF STATEMENT

Date: 26th Feb 2025

Description of work

To hoist down 1 case of 1 unit machine from lorry chassis to the ground, uncrate, using pallet truck to shift and position into production at ground floor

New Machine Spec

1 unit MWE6 Pro400 machine

Case dim: 225 x 190 x 220 cm

Wt: 2, 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

07/03/25

Time of Operation

10:00 am

Location of Operation

#### 10 Loyang Lane

Cyrus Technology

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar/woodblock

To lay out onto flooring for floor protection – 1 layer of canvas, 1 layer of plywood, 1 layer of PVC

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Using crowbar to uncrate crate parts by parts

- After uncrate, place all parts of crate to one side

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert pallet truck under machine bed

- To lay out 1 layer of canvas, 1 layer of plywood, 1 layer of onto flooring for floor protection

- Shift and transport machine into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175