# METHOD OF STATEMENT

Date: 11th Dec 2024

Description of work

To unload 2 unit machine from lorry chassis, shift and position into customer premise at 3rd floor via cargo lift

Machine Spec

2 unit Chamber 1000L

Dim: 223 x 129 x 245 cm each

Wt: 842 kg each

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/12/24

Time of Operation

09.45am

Location of Operation

#### 110 International Rd

Molex Singapore Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar

Woodblock

Canvas/PVC (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate machine case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert pallet truck under machine bed

- Shift and transport machine into factory premise at 3rd floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175