# METHOD OF STATEMENT

Date: 13th May 2025

Description of work

1) Lorry crane to hoist down case from lorry chassis to the ground, uncrate case, hoist and detach machine from base, using pallet truck to shift and position into production at ground

2) To shift out 1 unit machine to loading area, hoist up onto lorry chassis and deliver to scrapyard for disposal

Machine Spec

Move in

1 unit Hwacheon CNC Lathe / Cutex 180BL

Dim: 422 x 220 x 225 cm

Wt: 4, 750 kg

Move out to dispose

1 unit Deckol Maho DMU-80T

Wt: 8, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/05/25

Time of Operation

10:00 am

Location of Operation

#### 26 A.M.K Ind Pk 2

Meiban Mold Manufacture Pte Ltd

Equipment Use

1-2 x 633 lorry loader

Rollers/pallet truck

Wire sling

Webbing sling

Shackle

Hydraulic jack/woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Hoist down case from lorry chassis to the ground

- Uncrate case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down onto machine stand

- Insert rollers under onto machine bed

- Shift and transport into production at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175