# METHOD OF STATEMENT

Date: 04th Apr 2023

Description of work

To unload 4 packages from lorry chassis using forklift, uncrate, shift and position into factory premise at ground floor

Spec

1 set Modula Manual Lifter

Dim 1: 2800 x 2300 x 850 mm / Wt: 1, 415 kg

Dim 2: 3000 x 2300 x 2370 mm / Wt: 1, 508 kg

Dim 3: 2800 x 2300 x 1200 mm / Wt: 1, 160 kg

Dim 4: 2800 x 2300 x 1400 mm / Wt: 1, 012 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

04/04/23

Time of Operation

09:45 am

Location of Operation

#### 40 Lok Yang Way

Shine Precision

Equipment Use

1 x 633 lorry loader

1 x 3 ton forklift

Pallet truck

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                M.O.M certified forklift operator

# JOB SCOPE

- Using forklift to unload cases from lorry chassis

- Load cases to the ground

- Uncrate case

- Fork up and detach equipment from base

- Transport to near door entrance

- Using pallet truck to shift into factory premise

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175