# METHOD OF STATEMENT

Date: 02nd Oct 2023

Description of work

To hoist down machine from lorry chassis to the ground, using pallet truck to shift and position into factory premise

Machine Spec

1 unit Aero machine

Dim: 192 x 239 x 300 cm

Wt: 2, 500 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

21/10/23

Time of Operation

09:45 am

Location of Operation

#### 5 Tuas Dr 2

Turbine Overhaul Services Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

Wire/webbing sling

Crowbar/woodblock

Canvas/PVC sheet (to lay out for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down case from lorry chassis to the ground

- Uncrate machine case and put aside

- Attached sling to crane hook and secured to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175