# METHOD OF STATEMENT

Date: 02nd July 2024

Description of work

To unload 3 cases of 1 unit from lorry chassis to the ground, uncrate, using pallet truck to shift and position to 4th floor via cargo lift

Machine Spec

3 cases of 1 unit XO87-1000 Premium CMM

Case 1: 2280 x 1810 x 2100 mm / Wt: 2, 438 kg

Case 2: 1660 x 1430 x 920 mm / Wt: 200 kg

Case 3: 1400 x 1100 x 1030 mm / Wt: 127 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

15/07/24

Time of Operation

09:40 am

Location of Operation

#### 51 Marsiling Rd

Beyonics International Pte Ltd

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar/woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down onto machine stand

- Insert pallet truck under onto machine stand

- Shift and transport machine to 4th floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175