# METHOD OF STATEMENT

Date: 29th May 2025

Description of work

To unload 8 packages of 2 units machine from 1 x 40ft HQ & 1 x 20ft GP container, uncrate, shift and position into customer premise at ground floor

Machine Spec

8 packages of 2 Units Cut 200D Machines with Accessories

2 Packages Dimension: 280 x 232 x 256 cm – N/G Wt: 3, 820 kgs / 4,550 kgs

2 Packages Dimension: 187 x 242 x 202 cm – N/G Wt: 573 kgs / 875 kgs

2 Packages Dimension: 143 x 130 x 159 cm – N/G Wt: 145 kgs / 298 kgs

2 Packages Dimension: 120 x 95 x 95 cm – N/G Wt: 500 kgs / 600 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

03/06/25

Time of Operation

10:00 am

Location of Operation

No.51 Seletar Aerospace View

P&W NGPF Manufacturing Company Singapore Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x low bed

1 x 7 ton forklift

Rollers/pallet truck

Wire sling/webbing

Crowbar/hand jack

Woodblock

Canvas/metal sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

M.O.M certified forklift operator

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport lorry crane into factory premise

- Using forklift to fork down case from container to the ground

- Uncrate case and put aside

- Forklift to fork and detach machine from base

- Transport machine to door entrance

- Insert rollers/pallet truck under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert leveling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175