# METHOD OF STATEMENT

Date: 10th Feb 2022

Description of work

To unload 1 unit machine from container, uncrate, shift and position into factory premise

Machine Spec

1 unit machine

Dim: 4.5 x 2.3 x 2.3

Wt: 2, 700 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/02/22 (Sat)

Time of Operation

09:30 am

Location of Operation

#### 51 Seletar Aerospace View

Pratt & Whitney NGPF Manufacturing Company

Equipment Use

1 x 633 lorry loader

1 x forklift

Rollers/pallet truck

Canvas/metal sheet (to lay out for floor protection)

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry crane back to back with container

- Crane to pull out case from container onto lorry chassis

- Using forklift to fork down case to the ground

- Uncrate case and put aside

- Fork up and detach machine from base

- Transport machine to near door entrance

- Unload machine to the ground

- Using rollers/pallet truck to shift machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175