# METHOD OF STATEMENT

Date: 17th Oct 2024

Description of work

To unload 1 unit conveyor from lorry chassis, shift and position into customer premise at ground floor

Machine Spec

1 unit chip conveyor

Dim: 342 x 100 x 210 kg

Wt: 550 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

17/10/24

Time of Operation

Afternoon schedule

Location of Operation

#### 7 Benoi Crescent

Reda Production Systems

Equipment Use

1 x 633 lorry loader

Pallet truck

Webbing sling

Crowbar/woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Uncrate conveyor case and put aside

- Attached sling to crane hook

- Secured sling to rigging point of conveyor

- Hoist up conveyor away from base

- Swing crane and lower down conveyor to near ground level

- Insert pallet truck under conveyor bed

- Shift and transport into factory premise at ground floor

- Position at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175