# METHOD OF STATEMENT

Date: 25th Mar 2025

Description of work

To unload and hoist down 3 cases of 1 unit machine from low-bed/lorry chassis to the ground, uncrate, using rolllers to shift and position into factory premise

Machine Spec

3 cases of 1 unit CTX Beta 800 TC

Case 1: 460 x 280 x 288 cm / Wt: 9, 200 kg

Case 2: 550 x 170 x 214 cm / Wt: 1, 570 kg

Case 3: 210 x 210 x 210 cm / Wt: 750 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

07-08/04/25

Time of Operation

09:00 am

Location of Operation

#### 7 Joo Koon Circle

Honeywell Aerospace (S) Pte Ltd

Equipment Use

1-2 x 633 lorry loader

1 x low-bed

Rollers/pallet truck

Wire/webbing sling

Hydraulic jack

Woodblock

Canvas/PVC sheet (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Set up crane with outrigger fully extended and resting on timber mats as base plate

- Reverse low-bed to near crane

- Hoist down case from low-bed chassis to the ground

- Uncrate machine case and put aside

- Rigger to attached sling to crane hook

- Secured sling to rigging point of machine

- Hoist up and detach machine from base

- Swing machine away from base

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175