# METHOD OF STATEMENT

Date: 27th May 2022

Description of work

To hoist down 1 unit forklift from lorry chassis, using forklift to fork down case, uncrate, shift and position into factory premise at ground floor

Spec

1 unit Nanotom XCT

Dim: 200 x 93 x 150 cm

Wt: 2, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

09/06/22

Time of Operation

11:30 am

Location of Operation

#### 8 Cleantech Loop

Ceantech 3, NMC-Blk 1B (01-22C)

Equipment Use

1 x 633 lorry loader

1 x 3ton battery forklift

1 set wire/webbing sling

Pallet truck

Metal sheet/Plywood (to lay out for floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified lorry loader and forklift operator will be involved in this operation

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down forklift from lorry chassis to the ground

- Using forklift to fork down case from lorry chassis to the ground

- Uncrate case and put aside

- Fork up and detach machine from base

- Transport machine to near door entrance

- Unload machine to the ground

- To lay out metal sheet/plywood along pathway for floor protection

- Using pallet truck to shift machine into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175