# METHOD OF STATEMENT

Date: 26th Sep 2023

Description of work

To unload 36 wooden cases of 9 set machine from 7 x 40ft HQ container, uncrate, shift and position into production area at ground floor

Machine Spec

Refer to packing list attached

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate 2 day job

02-03/10/23

Time of Operation

09:45 am

Location of Operation

#### 91 Tuas Ave 1

SMC Manufacturing (S) Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x 4.5 ton forklift

Rollers/pallet truck

Canvas/metal sheet (to lay out for epoxy floor protection)

Wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader and forklift operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry back to back with container

- Crane to pull out machine cases from container onto lorry chassis

- Using forklift to fork down cases from lorry chassis to the ground

- Uncrate case and put aside

- Unscrew all nuts

- Forklift to fork up and detach machine from base

- Transport and unload machine at door entrance

- Insert pallet truck under machine base

- Transport machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175