# METHOD OF STATEMENT

Date: 16th Dec 2025

Description of work

Using lorry crane hoist down and upright machine, shift and position into factory premise, using forklift to fork up onto 40cm foundation

Machine Spec

1 unit Testing machine / Wt: 9, 600 kg

1 unit Testing machine / Wt: 3, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Estimate:

22/12/25 – 31/12/25

Time of Operation

10:00 am

Location of Operation

#### 35 Changi South Ave 2

Sunny Metal

Equipment Use

1 x 633 lorry loader

16ton forklift

Rollers/pallet truck

Wire sling

Shackle

Hydraulic jack

Woodblock

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

M.O.M certified forklift operator

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Reverse Malaysia to near lorry crane

- Insert sling to crane hook and secured to rigging point of machine

- Hoist down to the ground and upright machine

- Lower down machine to near ground level

- Insert rollers under machine bed

- Shift and transport machine into factory premise at ground floor

- Using 16ton forklift to fork up machine onto foundation

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175