# METHOD OF STATEMENT

Date: 25th Aug 2021

Description of work

1) To unload 1 case from lorry chassis to the ground using forklift, uncrate, using pallet truck to shift and position into premise at ground floor

2) To shift out 1 unit used CMM machine from production to loading area, fork up onto lorry chassis and transport to other location

Machine Spec

Move in

1 unit Tigo 565 CMM

Dim: 160 x 120 x 240 cm

Wt: 1, 100 kg

Move out

1 unit Tigo 565 CMM

Dim: 160 x 120 x 240 cm

Wt: 1, 100 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

01/09/21   /   09:45 am

Location of Operation

#### 6 Fusionopolis Way, Synthesis Building

Singapore Institute of Manufacturing Technology

Equipment Use

1 x 633 lorry loader

1 x forklift

Pallet truck/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry loader operator and forklift driver will be involved in this operation

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Hoist down forklift  from lorry chassis to the ground

- Uncrate case

- Forklift to fork and detach machine from base

- Transport and unload machine at loading bay

- Insert pallet truck under machine stand

- Shift and transport machine into building premise at ground floor

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175