# METHOD OF STATEMENT

Date: 11th June 2025

Description of work

To shift out 1 unit machine and accessories from basement to 2nd floor loading bay via cargo lift, using lorry crane to hoist up machine onto lorry chassis and deliver to new location, unload, shift and position into premise at ground floor

Machine Spec

1 x Hammer C3 41 Combination Saw / Wt: 800 kg

1 x Table

2 x Dust Collector

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

18/06/25

Time of Operation

09:30 am

Location of Operation

#### NUS

#### Move out: 18 College Ave West

#### Move in: 2 Engineering Dr 3, Blk E3 #B1-06

#### College of Design & Engineering, Central Workshop

Equipment Use

1 x 633 lorry loader

Pallet truck

Crowbar/woodblock

1 set webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Shift out machine and accessories from basement to loading area

- Rigger to insert sling to crane hook

- Secure sling to rigging point of machine

- Hoist up machine and load onto lorry chassis

- Secure machine with strapping belt

- Transport and deliver to new location

- Hoist down machine from lorry chassis to near ground level

- Insert pallet truck under machine bed

- Shift and transport into factory premise

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Kevin Wong / Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175