# METHOD OF STATEMENT

Date: 30th Mar 2023

Description of work

To unload 2 unit machine from lorry chassis using forklift, transport to loading bay, shift and position into customer premise at ground floor

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Depend on shipping arrival

11/04/23   -   29/04/23

Time of Operation

09:30 am

Location of Operation

#### Blk 3015A Ubi Rd 1 #01-09/10

Tech-Team Precision Toolings Pte Ltd

Equipment Use

2-3 x 633 lorry loader

1 x low bed

1 x platform

1 x 10 ton forklift

1 set rollers

Pallet truck

Obstacles

No obstacle on operation area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane and forklift operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated location

- Using 10 ton forklift to fork down machine from lorry chassis

- Transport machine to loading bay

- Lower down machine onto woodblock

- Insert rollers/pallet truck under machine base

- Shift and transport machine into tenant premise at ground floor

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and disposed

- Job complete

Best Regards

Zailani / Kevin Wong

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175