# METHOD OF STATEMENT

Date: 16th Jan 2019

Description of work

Using forklift to unload 3 cases from lorry chassis, uncrate, shift and position into customer premise at ground floor

Machine Spec

1 set of STC Vibration Test System Amplifier

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

22/01/2019   –   09:30am

Location of Operation

#### 39 Changi North Crescent

Collins Aerospace

Equipment Use

3 x lorry crane

Rollers/pallet truck

1 x 4.5 ton forklift

Metal sheet/plywood (for rubber tiles floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)

M.O.M certified forklift operator

# JOB SCOPE

- Forklift to unload cases from lorry chassis

- Uncrate case

- Fork up and detach machine from base

- Transport machine to entrance door

- Lower down machine onto wooden block

- Insert rollers/pallet truck to machine base

- Shift machine into production area at ground floor

- Position machine at the designated location

- Unload machine from rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175