# METHOD OF STATEMENT

Date: 22nd Dec 2017

Description of work

To hoist down 1 unit machine with accessories from low loader @ unloading area, insert rollers to machine base, shift and position into production area at ground floor

Machine Spec

1 unit Mazak INTE J-200

Dim: 463 x 271 x 295 cm

Wt: 9, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

28/12/2017   –   13:00 pm

Location of Operation

#### No 1 Bedok South Rd

Panasonic Appliances Refrigeration Devices Singapore

Equipment Use

1 x 50 ton lorry loader

1 x low loader

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry loader to park at designated unloading area

- Reverse low loader to near crane

- Hoist up machine from low loader

- Lower down machine to near ground level

- Insert rollers to machine base

- Transport machine into premise

- Shift and position to the designated location

- Unload machine from rollers and insert plate under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175