# METHOD OF STATEMENT

Date: 18th Dec 2020

Description of work

To hoist down 1 case from lorry chassis to the ground, uncrate, using pallet truck to shift and position machine to 5th floor via cargo lift

Machine Spec

1 unit Global Explorer Performance 05.07.05 CMM
Case Dim: 3038 x 1288 x 2239 mm

Wt: 1, 118 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

29/12/2020 (Tuesday)

Time of Operation

09:45 am

Location of Operation

2 Changi North St 2

ETLA

Equipment Use

1 x 633 lorry loader

1 set webbing sling

Pallet truck

Plywood (to lay out till outside of clean room)

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated unloading area

- Insert sling under machine case

- Hoist down case from lorry chassis to the ground

- Uncrate case

- Attached required lifting gears onto crane hook and secured to rigging point of machine to be lifted

- Hoist up machine 1ft above ground to check on balancing

- When well balanced, hoist up and swing crane away from base

- Lower down machine onto machine stand

- Insert pallet truck to machine stand

- Transport machine to 5th floor via cargo lift

- Position machine at the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175