# METHOD OF STATEMENT

Date: 25th Sep 2019

Description of work

1) To shift out and hoist down 2 unit machine from 3rd floor using lorry crane, load onto Malaysia lorry and deliver to new location

2) To shift out 7 unit small machine from 3rd floor to ground floor via cargo lift and load onto Malaysia lorry

Machine Spec

1 unit Seyi SN1-25 Press

Dim: 160 x 90 x 210 cm   /   Wt: 2000 kg

1 unit FCB 21 Press

Dim: 90 x 70 x 230 cm   /   Wt: 1200 kg

Risk Assessment

A copy of Risk Assessment will be attach

Date of Operation

01/10/2019

Time of Operation

09:45 am

Location of Operation

#### FPC Components Pte Ltd

Blk 1023 Yishun Industrial Pk A #03-05/07

Equipment Use

1 x 633 lorry loader – PALFINGER

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Barricade work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Shift out machine from production to hoisting area

- Insert sling to machine hoisting point and hook onto crane

- Hoist up 1ft above ground to check on balancing

- When well balanced, hoist down machine slowly to the ground

- Load machine onto Malaysia lorry

- Secure machine with strapping belt prior to moving out of premise

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175