# METHOD OF STATEMENT

Date: 22nd Mar 2018

Description of work

To shift out and hoist down 1 unit machine from 4th floor to ground floor for packing by using lorry crane

Machine Spec

1 unit Boy 60E Injection Molding machine

Dim: 325 x 110 x 165 cm

Wt: 2, 600 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

23/03/2018   –   09:30 am

Location of Operation

#### Injection Engineering

Blk 1023 Yishun Industrial Pk A #04-07

Equipment Use

1 x 45 ton lorry crane

1 x lorry crane

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine from production to hoisting area

- Insert sling to machine hoisting point

- Lorry crane to hoist down machine from 4th floor to the ground

- Load machine onto wooden base for packing

- After packing, stuff case into container

- Secure machine with lashing wire and choking wood

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175