# METHOD OF STATEMENT

Date: 16th Oct 2017

Description of work

To unload estimate 50-55 units Conveyor from 10-11 x 40ft GP/HQ container, shift and position into factory near trench, assist engineer to assemble and fixed conveyor, and lower down into trench @1.8m depth

Risk Assessment

A copy of Risk Assessment will be attached

Start/Completion Date

As per required schedule

Time of Operation

As per required schedule

Location of Operation

#### 12 Jalan Tukang Innovation

Dorma Production GmbH & Co KG

Equipment Use

1-2 x lorry crane

Forklift

Pallet truck

1 ton chain block x 6 pcs

7m webbing sling x 10 pcs

1.25 ton U shackle x 20 pcs

25 kg Level Block x 6 pcs

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman, lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Flush lorry crane to level with container

- Crane to pull out conveyor from container and load onto chassis

- Forklift to unload conveyor from lorry chassis

- Shift and position conveyor into factory near trench

- To assist engineer assemble and fixed 5 set of conveyor at 1 time

- After fixing, using webbing sling and 5 x forklift to lift up conveyor

- Shift and position conveyor into trench

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remarks

MOS subject to change by supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175