# METHOD OF STATEMENT

Date: 27th Oct 2022

Description of work

To unload 4 cases of 2 units machine from container to the ground, uncrate, shift and position into factory premise at ground floor

Machine Spec

3 Packages of 1 unit Fast Speed Wirecut EDM Machine

Dim: 170 x 200 x 230cm/ Wt: 3,500kgs

Dim: 60 x 60 x 90cm/ Wt: 150kgs

Dim: 60 x 40 x 40cm/ Wt: 80kgs

1 Package of 1 unit EDM Drilling

Dim: 210 x 210 x 230cm/ Wt: 600kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

28/10/22

Time of Operation

10:00 am

Location of Operation

#### 127 Pioneer Rd

Fong Lee Metal Industries Pte Ltd

Equipment Use

1 x 633 lorry loader

Heavy duty rollers/pallet truck

1 set wire/webbing sling

Shackle

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry loader at designated location

- Insert wire sling under machine case

- Hoist up machine case to check on balancing

- When well balancing, hoist down machine case from container

- Unload case to the ground

- Uncrate case

- After uncrate, hoist machine and detach from base

- Swing machine away from base

- Lower down machine slowly to near ground level

- Insert rollers to machine base

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175