# METHOD OF STATEMENT

Date: 09th Apr 2021

Description of work

To unload 1 unit machine and parts from 1 x 40ft FR and 1 x 20ft GP container, uncrate, shift and position into factory premise at ground floor

Machine Spec

Doosan PUMA 700

Dim: 5760 x 3145 x 2780 mm

Wt: 16, 300 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

Est: 19/04/21 (Monday)

Time of Operation

09:45 am

Location of Operation

#### No 149 Gul Circle

FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 50 ton mobile crane

633 lorry loader

Heavy duty rollers

Wire sling

Webbing sling

Shackle

1 x forklift

1 x scissor lift

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator and forklift driver will be involved in this operation

Safety Aspect

Access to be restricted to the site area, be cordoned off with safety cones, red/white tape and lifting in operation signage

Assignment of traffic controller to control and guide movement of vehicle and personnel within the premise

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy and set up mobile crane at designated location for the lift

- Insert sling under machine case

- Hoist up machine case to check on balancing

- When well balanced, hoist down case from container

- Unload case to the ground

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to near unloading area

- Lower down machine to near ground level

- Insert heavy duty rollers to machine base

- Using forklift to assist in pushing/pulling machine into factory production area

- Position machine at the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani / Kevin

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175