# METHOD OF STATEMENT

Date: 05th June 2018

Description of work

To unload 1 unit machine from 1 x 4axle Malaysia low bed and 3 x 40ft Malaysia lorry, shift and position into customer premise at ground floor

Machine Spec

1 unit GEMINIS machine

Main Bed Dim: 34ft x 6ft x 7.5ft

Wt: 40,000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBC

Location of Operation

#### No 149 Gul Circle

FMC Technologies Singapore Pte Ltd

Equipment Use

1 x 200 ton mobile crane

Lorry crane

Heavy duty rollers

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Reverse low bed to near crane

- Insert sling to machine hoisting point

- Hoist up machine from low bed and lower down to the ground

- Swing crane to loading area

- Insert heavy duty rollers to machine base

- Using forklift to assist in pushing/pulling machine into production area

- Shift and position machine to the designated location

- Unload machine from rollers and insert levelling pad under machine bed

- To unload machine parts from Malaysia lorry and transport to location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175