# METHOD OF STATEMENT

Date: 08th Apr 2019

Description of work

To unload 4 cases of 2 unit machine from lorry chassis, uncrate, shift and position into customer production area

Machine Spec

Mazak Horizontal Machining Center/HCN 4000

Case Dim: 485 x 239 x 299 cm

Wt: 11, 000 kgs each

Risk Assessment

A copy of Risk Assessment will be attached

Date/Time of Operation

08/04/2019   -   09:30 am

Location of Operation

#### No 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 50 ton lorry crane

1 x lorry crane

1 x forklift

Heavy duty rollers/pallet truck

1 set wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Transport of lorry crane to loading/unloading area

- Unload and hoist down cases from lorry chassis to the ground

- Uncrate machine case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Insert rollers to machine base

- Use of forklift to assist in pulling/pushing to transport machine into production

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175