# METHOD OF STATEMENT

Date: 29th Jan 2018

Description of work

To unload 1 unit machine from 1 x 20ft GP container, uncrate, shift and position into customer premise

Machine Spec

1 unit PSG-C60150AHR

Dim: 4850 x 2500 x 2040

Wt: 7, 500 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

31/01/2018   –   09:30am

Location of Operation

#### No 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 45/50 ton lorry crane

Heavy duty rollers/pallet truck

Metal plate/canvas (for epoxy floor protection)

Wire/webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Pull out machine case from container and load onto lorry chassis

- Transport lorry to unloading area

- Hoist down case from lorry chassis

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to loading area

- Lower down machine to near ground level

- Insert rollers to machine base

- Transport machine into production room

- Shift and position to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175