# METHOD OF STATEMENT

Date: 22nd July 2021

Description of work

To unload 1 unit machine from 1 x 20ft container, uncrate, shift and position into factory premise at ground floor

Machine Spec

1 unit E Tech EGM-350 Grinding

Dim: 391 x 228 x 200 cm

Wt: 5, 800 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

22/07/21

Time of Operation

09:45 am

Location of Operation

#### No 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 633 lorry loader

Rollers/pallet truck

1 set wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Reverse lorry to near container (back to back with container)

- Pull out case from container onto lorry chassis

- Hoist down case to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane to unloading area

- Insert rollers/pallet truck under machine bed

- Transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers/pallet truck and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175