# METHOD OF STATEMENT

Date: 09th July 2021

Description of work

To unload 2 cases from low loader, uncrate, shift and position into factory premise at ground floor

Machine Spec

Mazak CNC Vertical i-700T

Dim: 264 x 312 x 343 cm

Wt: 16, 000 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

12/07/21

Time of Operation

10:00 am

Location of Operation

#### No 16 Seletar Aerospace Crescent

JEP Precision Engineering Pte Ltd

Equipment Use

1 x 75 ton lorry loader

1 x 633 lorry loader

1 x low-loader

1 x forklift

Heavy duty rollers/pallet truck

1 set wire/webbing sling

Metal plate/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade off operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry loader operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Reverse low loader to near crane

- Hoist up cases from low loader

- Unload case to the ground

- Uncrate case

- Hoist up and detach machine from base

- Swing crane away from base and lower down to near ground level

- Insert rollers to machine bed

- Use of forklift to assist in pulling/pushing to transport machine into production area

- Position machine at the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175