# METHOD OF STATEMENT

Date: 25th Sep 2018

Description of work

To hoist up 1 unit new Motor to 7th floor roof top and hoist down 1 unit used Motor from 7th floor roof top to ground floor

Motor Spec

Dim: 121cm x 60cm

Wt: 265 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date Of Operation

29/09/2018

Time Of Operation

09:30 am

Location of Operation

#### 18 Woodlands Loop

Edmund Optics Singapore Pte Ltd

Equipment Use

1 x 50 ton mobile crane

Wire sling/shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and mobile crane operator will be involved in this operation

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Mobile crane to park at designated hoisting area for the lift

- Attached sling and shackle to motor hoisting point

- Mobile crane to hoist up new motor to 7th floor roof top

- Position motor at the designated location

- To hoist down old motor from 7th floor roof top to the ground

- Position motor to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be dispose and clear

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175