# METHOD OF STATEMENT

Date: 29th Mar 2018

Description of work

To shift out and hoist down 1 unit machine from 2nd floor to ground floor by using lorry crane, shift and position into #01-29

Machine Spec

1 unit CMM 9.15.8

Dim: 1750 x 2450 x 2310 mm

Wt: 3, 560 kgs

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

05/04/2018   –   Afternoon slot

Location of Operation

#### 2 Woodlands Sector 1 #01-29

Precision Products Singapore Pte Ltd

Equipment Use

1 x 50/45 ton lorry crane

Pallet truck

Webbing sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet, safety harness etc..)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Lorry crane to park at designated hoisting area for the lift

- Shift out machine to near shutter down opening

- Insert webbing sling to machine hoisting point

- Lorry crane to hoist down machine from 2nd to ground floor

- Shift and position machine to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175