# METHOD OF STATEMENT

Date: 06th Oct 2020

Description of work

To unload 7 packages of 1 unit machine from customer arranged container, hoist up to 3rd floor by using platform and lorry crane

Risk Assessment

A copy of Risk Assessment will be attach

Date / Time of Operation

06/10/20   -   Afternoon schedule

Location of Operation

#### 2 Woodlands Sector 1

ST Engineering Aerospace Precision Products Pte Ltd

Equipment Use

2 x 633 lorry loader

1 x platform

Rollers/pallet truck

Wire sling

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and disposed

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation

Safety Aspect

Assignment of traffic controller to control and guide movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in progress signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness)

Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry loader at designated hoisting area for the lift

- Hoist up platform to 2nd level opening

- Stabilize platform and ensure it is fully rested on the opening slab

- Tied back platform to a competent point and secured using 50 ton lorry crane

- Use strong belt to secure front left and right of platform onto building pillar to prevent platform swing outward

- Lorry loader to hoist up machine to 3rd floor

- Lower down machine onto platform

- Insert rollers onto machine base

- Shift out machine from platform and position to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175