# METHOD OF STATEMENT

Date: 06th Aug 2020

Description of work

To unload 16 cases of 1 unit machine from customer arrange container using factory overhead crane, uncrate, shift and position to location

Machine Spec

You Ji VTL 2500 CNC

Column: 5.45 x 4.05 x 2.82 m

Wt: 30, 000 kg

Table: 5 x 4.2 x 2.62 m

Wt: 30, 000 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

TBA

Time of Operation

TBA

Location of Operation

#### 22 Jalan Terusan

FMC Facility

Equipment Use

1 x 633 lorry loader

Factory 38 ton overhead crane

Heavy duty rollers

Wire/webbing sling

Shackle

Shackle

Obstacles

No obstacle on route of transportation

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Deploy lorry crane at designated unloading area

- Insert wire sling under machine case

- Hoist down machine case from FR container to the ground

- Uncrate machine case

- Hoist up machine and load onto lorry chassis

- Transport lorry crane with machine into inside factory

- Using customer overhead crane to unload machine

- Position machine at the designated location

- Lower down and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175