# METHOD OF STATEMENT

Date: 25th Apr 2019

Description of work

To unload Machineries and move into customer factory

Risk Assessment

A copy of Risk Assessment will be attached

Date of Operation

26/04/2019 – 27/04/19

Time of Operation

10:00 am – 7:00 pm

Location of Operation

25B Loyang Crescent, Loyang Offshore Supply Base,

Blk 302 #01-18

Equipment Use

1 x 50 ton mobile crane

1 x 50 ton lorry crane

1 set wire sling

1 set shackle

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carry out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crew certificate)

Safety Aspect

Assignment of traffic controller to control movement of vehicle and personnel within the vicinity

Barricade off work area with safety cones, red/white tape and lifting in operation signage

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, safety helmet, safety harness) Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman.

# JOB SCOPE

- 1 x Mobile 1 x Lorry crane to park at designated hoisting area for the lift

- Position Trailer at hoisting area

- Setup Wire Rope and Shackle to hoist down Machinery

- Lift onto ground

- Use both Cranes to hoist the machine up

- After Hoisting away from ground lorry crane lower for Mobile crane to Upright the Machinery

- After Upright, insert rollers underneath the machinery

- Position machinery at the designated location inside factory

- Once job complete, team leader will instruct the team to perform proper housekeeping

- All debris will be clear and dispose

- Job complete

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175