# METHOD OF STATEMENT

Date: 09th Jan 2017

Description of work

To unload 7 unit Mazak machine from M’sia low loader/lorry, shift and position into customer premise at ground level

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

Est: 11/01/2017   -   To unload 1 unit Mazak QTN350M, 1 unit Mazak Slant 50NX-3000U and 1 unit Mazak Slant Turn Nexus 500

Est: 12/01/2017   -   To unload 1 unit Mazak Slant Turn Nexus 500M and 1 unit Mazak Slant Turn Nexus 550

Est: 14/01/2017   -   To unload 1 unit Mazak Intergrex E650H and 1 unit Mazak ITE-E410H2

Location of Operation

#### No 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte Ltd

Equipment Use

1 x 70 ton mobile crane

1 x 100 ton mobile crane

Lorry crane

Heavy duty rollers

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on crane parking area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and mobile/lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by MOM certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of mobile crane at designated location for the lift

- Hoist down machine from M’sia low loader and load onto ground

- Uncrate machine case

- Hoist and detach machine from base

- Swing machine to loading area

- Insert heavy duty rollers to machine base

- Using forklift to assist in pushing/pulling and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175