# METHOD OF STATEMENT

Date: 26th Oct 2017

Description of work

1) To pull out 1 unit Torque machine from container, insert rollers to machine base, shift and position into production area at ground floor

2) To shift out 1 unit Clincher machine from production area, load onto container, and to secured machine with choking wood and lashing belt/wire

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

30-31/10/2017   -   10:00 am

Location of Operation

#### No 273 Jalan Ahmad Ibrahim

Baker Hughes Singapore Pte Ltd

Equipment Use

1 x lorry crane

Heavy duty rollers/pallet truck

Wire/webbing sling

1 x Forklift

Obstacles

No obstacle on loading/unloading area and route to machine position

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Qualified forklift operator

# JOB SCOPE

- Pull out machine from container chassis using forklift/lorry crane

- Insert rollers to machine base

- Using forklift to assist in pushing/pulling and position machine to the designated location

- Jack up machine, pull out rollers and insert levelling pad under machine bed

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris, if any will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175