# METHOD OF STATEMENT

Date: 30th April 2018

Description of work

To unload 1 unit machine from 1 x container, uncrate, shift and position into customer premise

Machine Spec

1 New Heat Exchanger

Dim: 347 x 197 x 229 cm

Wt: 2, 490 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

TBC

Location of Operation

#### No 8 Pandan Ave

Boon Heng Engineering Pte Ltd

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman, lorry crane operator and forklift operator will be involved in this operation

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up lorry crane at designated location (back to back with container)

- Pull out cases from container and load onto lorry chassis

- Hoist down case to the ground

- Uncrate case

- Hoist and detach machine from base

- Swing crane to unloading area

- Insert rollers/pallet truck to machine base

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark:

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175