# METHOD OF STATEMENT

Date: 06th March 2018

Description of work

To unload 3 cases from lorry chassis, uncrate, shift and position into customer premise at ground floor

3 cases of 1 unit machine

Case 1 Dim: 98 x 98 x 42 cm   /   Wt: 77 kgs

Case 2 Dim: 126 x 106 x 99 cm   /   Wt: 156 kgs

Case 3 Dim: 136 x 96 x 203 cm    /   Wt: 647 kgs

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

12/03/2018   -   09:45 am

Location of Operation

#### 3 Cleantech Loop #01-01, Clean Tech 2

Advanced Remanufacturing & Technology Centre

Equipment Use

1 x lorry crane

Pallet truck

Webbing sling

Plywood/canvas (for epoxy floor protection)

Obstacles

No obstacle on crane parking area

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

MOM certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location

- Load down cases from lorry chassis onto ground

- Uncrate case

- Hoist and detach machine from base

- Swing machine to unloading area

- Load machine onto pallet truck

- Shift and position machine to the designated location

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175