# METHOD OF STATEMENT

Date: 03rd Apr 2018

Description of work

To unload 2 cases of 1 unit machine from 1 x 40ft HQ container, uncrate, shift and position into customer premise at ground level

Machine Spec

NLX 2500 CNC Lathe

Case 1: 4510 x 2280 x 2500 mm   /   Wt: 7, 050 kg

Case 2: 1410 x 1120 x 1780 mm   /   Wt: 120 kg

Risk Assessment

A copy of Risk Assessment will be attached

Date / Time of Operation

11/04/2018   –   09:45 am

Location of Operation

#### ARTC @ 3 Cleantech Loop #01-01

#### JTC Cleantech 2

Equipment Use

1 x lorry crane

Rollers/pallet truck

Wire/webbing sling

Canvas/metal sheet (for epoxy floor protection)

Obstacles

No obstacle on crane parking area and route to machine positioning

Barricade operation area to prevent persons who are not involved enter unintentionally

Environment

No lifting operation will be carried out on heavy rain, thunderstorms and lightning weather

All debris will be clear and dispose

Lifting Crew

M.O.M certified lifting supervisor, rigger, signalman and lorry crane operator will be involved in this operation (attached all lifting crews certificate)

Safety Aspect

All personnel have to ensure that risk control are known and supervisor has to ensure that risk control are in placed                                                                                                                            All our workers are fully equip with standard P.P.E. (safety shoes, helmet etc..)                                                                                                                                                Lifting machine and lifting gear with M.O.M certificate

Lifting operation are supervise by M.O.M certified lifting supervisor, rigger and signalman

# JOB SCOPE

- Setting up of lorry crane at designated location (back to back with container)

- Crane to pull out case from container onto lorry chassis

- Hoist down case from lorry chassis onto ground

- Uncrate case

- Hoist up and detach machine from base

- Swing machine to unloading area

- Lower down machine slowly to near ground level

- Insert rollers to machine base and secured with bolts and nuts

- Shift and position machine to the designated location at ground floor

- Once job complete, team leader will instruct the team to perform proper housekeeping, all debris will be clear and dispose

- Job complete

Remark

MOS subject to change by supervisor/lifting supervisor on site

Best Regards

Zailani

Eric Wong Machinery Transportation Pte Ltd

Tel: 62824175